# Claude Code Macro Pad Product Specification · v1.1 Fan project · Small batch · Community pre-order model · JLCPCB manufacturing

## Concept Overview

A themed mechanical macro pad for Claude Code power users. Robot-face silhouette, Claude orange matte case, black keycaps and knobs. Inspired by an X post that proved demand (100k+ views) but never shipped — this is the version that gets built.

## Colour Specification

| Element | Hex | RGB | Finishing Notes |
| --- | --- | --- | --- |
| Case | #DA7756 | 218 / 119 / 86 | Pantone 7416 C · SLS prints white — dye with Rit DyeMore (Apricot + Scarlet mix) post-print |
| Keycaps | #1A1A1A | 26 / 26 / 26 | Off-the-shelf black PBT or ABS blanks — no special finishing needed |
| Encoder knobs | #1A1A1A | 26 / 26 / 26 | Black anodised aluminium knurled knobs — standard M6 shaft fitting |
| Cable | #1A1A1A | 26 / 26 / 26 | Black braided USB-C — as shown in render |

Note on colour matching: JLCPCB SLS nylon is only available in white/natural. Post-print dyeing with fibre-reactive or nylon-compatible dye (Rit DyeMore) is the recommended finishing method. Dye soaks into the nylon rather than coating it, preserving the matte tactile texture. Batch-to-batch consistency is achievable with a measured dye recipe.

## Physical Dimensions

Derived from MX-compatible switch spacing (19.05 mm centre-to-centre), EC11 rotary encoder footprint, and 3 mm SLS nylon wall thickness. All measurements in millimetres.

| Dimension | Value | Derivation |
| --- | --- | --- |
| Overall footprint (incl. arms + leg notches) | 122 mm × 67 mm | Main body + 2× side arm keys + bottom notch profile |
| Main body (rectangular core) | 78 mm × 57 mm | 3.75U bottom row (71.4 mm) + 3 mm walls each side |
| Side arm extension (each) | 22 mm wide | 1U key spacing (19.05 mm) + 3 mm outer wall |
| Case depth (total height profile) | 17 mm | Base 2 mm + PCB 1.6 mm + MX switch 11.6 mm + top plate 1.5 mm + tolerance 0.3 mm |
| Encoder knob protrusion above case | ~18 mm | Standard EC11 shaft + 15 mm tall knurled aluminium knob |
| Key switch cut-outs | 14.5 mm × 14.5 mm | Standard MX plate cut-out + 0.25 mm clearance each side |
| USB-C port cut-out | 10 mm × 4 mm | Centred on top edge of main body |
| PCB bounding box | 116 mm × 51 mm | Case interior minus 3 mm walls; custom non-rectangular silhouette |
| Encoder centre-to-centre spacing | 38 mm | 2× key spacing — symmetric about vertical centreline |
| Recommended wall thickness (SLS) | 3 mm minimum | SLS nylon structural minimum; 3.5 mm recommended at arm joints |

## Bill of Materials

| Component | Qty | Spec | Source |
| --- | --- | --- | --- |
| Case (top + bottom) | 1 set | SLS PA12 nylon, white — dye to #DA7756 | JLCPCB 3D print |
| Custom PCB | 1 | Custom silhouette, 1.6 mm FR4, ENIG finish | JLCPCB PCB + PCBA |
| Pro Micro controller | 1 | ATmega32U4, USB-C preferred, drop-in | LCSC / AliExpress |
| Hotswap sockets | 5 | Kailh MX hotswap — PCB-mount | JLCPCB PCBA or pre-sourced |
| Diodes | 5 | 1N4148 SOD-123 SMD | JLCPCB PCBA |
| Rotary encoders | 2 | EC11 with detents, 20 mm shaft | LCSC |
| Encoder knobs | 2 | Black anodised aluminium, knurled, M6 | AliExpress / custom |
| Keycaps — 1U | 4 | Black PBT or ABS blank, MX stem | KBDFans / AliExpress |
| Keycap — 1.75U | 1 | Black PBT or ABS blank, MX stem (caps lock size — no stabiliser required) | KBDFans / AliExpress |
| MX switches | 5 | Buyer's choice — hotswap, any MX-compatible | Included or buyer-supplied |
| Braided USB-C cable | 1 | Black, ~1.5 m, USB-C to USB-A/C | Sourced separately |
| M2 screws | 4 | M2 × 6 mm, countersunk, black | Standard hardware |
| Brass heat-set inserts | 4 | M2 × 3.5 mm, for case assembly | Standard hardware |

## Production Path — JLCPCB

| PCB design | EasyEDA (native JLCPCB tool). Pro Micro footprint, 5 switch positions (4× 1U + 1× 1.75U), 2× EC11 encoder footprints, single-layer routing sufficient for this key count. Export Gerbers directly from EasyEDA to JLCPCB order. |
| --- | --- |
| PCBA assembly | Submit BOM + CPL files. Have JLCPCB populate hotswap sockets, diodes, EC11 footprints, and passive components. Pro Micro drops in after delivery — no SMD soldering required by hand. |
| Case 3D print | SLS PA12 nylon via JLCPCB 3D printing (same order). White/natural output. Wall minimum 3 mm. Specify 0.2 mm clearance on all mating faces. Top plate + base plate printed separately and assembled with M2 heat-set inserts. |
| Colour finishing | Post-print dye batch in Rit DyeMore. Mix Apricot + small amount of Scarlet to reach #DA7756 (218/119/86). Document dye ratios for batch consistency. Rinse, dry, optional thin matte clear coat for durability. |
| Firmware | QMK — well-supported for Pro Micro macropads with EC11 encoders. Flash via QMK Toolbox. Encoder map: left knob = volume/scroll, right knob = configurable. All keys user-remappable via VIA if VIA support is added. |

## Go-to-Market Strategy

1  Align with the original creator

DM them on X before building. Get explicit agreement — minimum: free unit + credit. Ideal: small per-unit revenue share so they have reason to actively promote, not just passively allow.

2  Build the prototype

Complete CAD, order prototype PCB + case from JLCPCB, dye the case to #DA7756. Goal: one physical unit to photograph. This is your entire marketing asset.

3  Coordinated reveal

Ship the prototype to the original creator. Agreed post: "I designed this — someone actually built it. Pre-orders open." Their existing audience becomes your launch audience.

4  Pre-order window

Take pre-orders via Shopify or equivalent. Set a firm MOQ and delivery timeline. Pre-order revenue fronts the manufacturing cost — no capital risk.

5  Production + fulfilment

Hit MOQ → full JLCPCB production order → assemble (drop in Pro Micro, flash firmware, fit switches + keycaps) → ship.

## Prioritised Next Steps

- DM the original X creator — align before committing time to CAD

- Finalise case CAD — robot silhouette, 3 mm walls, encoder + switch cut-outs, M2 insert bosses

- Design PCB in EasyEDA — Pro Micro footprint, 5× Kailh sockets, 2× EC11 footprints, USB-C passthrough

- Order JLCPCB prototype — PCB + PCBA + SLS case in a single order to test fit and finish

- Dye test — validate Rit DyeMore ratio to match #DA7756 before committing to batch

- Photograph finished prototype — this single asset drives all pre-order conversion

- Set up pre-order storefront — clear MOQ, timeline, and price listed upfront

Fan project — not affiliated with or endorsed by Anthropic. Claude and Claude Code are trademarks of Anthropic, PBC.