# Claude Code Macro Pad Product Spec & Go-to-Market Plan Fan project · Small batch · Community pre-order model

## Concept Overview

A themed mechanical macro pad designed for Claude Code power users. Robot-face silhouette, Claude orange matte case, black keycaps and knobs. Validated by an existing X post with 100k+ views that never shipped — this is the version that actually gets built.

## Materials Specification

| Component | Spec | Notes |
| --- | --- | --- |
| Case | SLS nylon or FDM plastic, matte finish | Claude orange — dye or pre-coloured filament |
| Keycaps | Black PBT or ABS blanks | 4× 1U + 1× 1.75U (no stabilisers required) |
| Switches | Hotswap sockets (Kailh) | Buyer's choice of switch |
| Encoders | 2× rotary encoders | Black metal knurled knob |
| PCB | Custom, Pro Micro footprint, QMK | Designed for the robot silhouette layout |
| Controller | Pro Micro (drop-in) | Simplifies assembly; no SMD soldering |
| Connection | Braided USB-C cable | As shown in render |

## Production Path

| PCB | Custom layout via EasyEDA → manufactured + PCBA assembled by JLCPCB. Hotswap sockets and passives populated by their assembly service. |
| --- | --- |
| Case | SLS nylon printed via JLCPCB 3D printing service. Dye with Rit DyeMore (Tangerine or similar) for consistent Claude orange across the batch. Single supplier for PCB + case. |
| Keycaps | Off-the-shelf black PBT blanks. Standard 1U and 1.75U sizes — no custom tooling or MOQ headaches. |
| Firmware | QMK — well-documented for Pro Micro macropads with rotary encoders. Open source, user-configurable. |

## Go-to-Market Strategy

1  Reach out to the original creator

DM them on X before building anything. Acknowledge their post drove the demand. Agree on a collaboration arrangement — at minimum, free unit + credit. Ideally a small revenue share so they have skin in the game and actively promote.

2  Build the prototype

Complete CAD for the case, lay out the custom PCB in EasyEDA, order a prototype run from JLCPCB. Goal: one physical unit in Claude orange with black keycaps and metal knobs.

3  Coordinated reveal

Ship the prototype to the original creator. They post the reveal to their existing audience: "I designed this, someone actually built it — pre-orders open." Their 100k-view base becomes your launch audience.

4  Pre-order window

Take pre-orders to front the manufacturing cost. No capital risk. Set a realistic MOQ and delivery timeline. Use Shopify or similar for a clean checkout experience.

5  Manufacturing run

Hit MOQ → place full production order with JLCPCB → assemble and ship.

## Prioritised Next Steps

- DM the original X creator — get alignment before committing time to CAD

- Finalise case design in CAD — nail the robot silhouette, wall thickness for SLS, encoder and switch cutouts

- Design PCB in EasyEDA — Pro Micro footprint, 5 switch positions (4× 1U + 1× 1.75U), 2× encoder footprints

- Order prototype from JLCPCB — PCB + PCBA + SLS case in one order

- Photograph finished prototype — this is your pre-order launch asset

- Set up pre-order storefront — Shopify or equivalent, with clear MOQ and timeline

Fan project — not affiliated with or endorsed by Anthropic. Claude and Claude Code are trademarks of Anthropic.