# Claude Code Macro Pad Product Specification · v1.3 Fan project · Small batch · Community pre-order model · JLCPCB manufacturing

## Concept Overview

A themed mechanical macro pad for Claude Code power users. Robot-face silhouette, Claude orange matte case, black keycaps and knobs. Inspired by an X post that proved demand (100k+ views) but never shipped — this is the version that gets built.

## Colour Specification

| Element | Target Hex | RGB | Colour Notes |
| --- | --- | --- | --- |
| Case | #DA7756 | 218 / 119 / 86 | Pantone 7416 C · FDM ASA printed in closest available orange/red-orange filament. Order a colour test print before committing to batch — compare JLCPCB orange and red-orange ASA swatches against #DA7756 in natural light. |
| Keycaps | #1A1A1A | 26 / 26 / 26 | Off-the-shelf black PBT or ABS blanks — no special finishing needed |
| Encoder knobs | #1A1A1A | 26 / 26 / 26 | Black anodised aluminium knurled knobs — standard M6 shaft fitting |
| Cable | #1A1A1A | 26 / 26 / 26 | Black braided USB-C — as shown in render |

Note on colour matching: FDM ASA filament is the only JLCPCB 3D printing technology offering multiple colours without post-processing. Their standard orange ASA skews brighter than #DA7756 — their red-orange option may land closer to Claude's terracotta. A cheap flat test print in both colours is the fastest way to confirm before ordering the full case.

## Physical Dimensions

All dimensions derived from actual component specs: MX switch spacing (19.05 mm c-to-c), EC11 encoder footprint, Kailh hotswap socket, OEM keycap profile, and FDM ASA 2.5 mm wall thickness. Verified against comparable real-world macro pads (Rama M6-B: 86 × 51 × 26 mm; Stream Deck Mini: 84 × 84 × 28 mm).

### Footprint  (X–Y)

| Dimension | Value | Derivation |
| --- | --- | --- |
| Overall width (incl. side arms) | 120 mm | Main body 76 mm + 2× arm (19.05 + 2.5 mm wall = 21.6 mm each) |
| Overall depth / height (incl. leg notches) | 66 mm | Main body 54 mm + 12 mm decorative leg notch profile |
| Main body (rectangular core) | 76 mm × 54 mm | 3.75U key span (71.4 mm) + 2.5 mm walls; encoder zone + key row + walls |
| Side arm protrusion (each) | 21.6 mm wide | 1U key spacing (19.05 mm) + 2.5 mm outer wall |
| Leg notch depth (bottom) | 12 mm | Decorative only — 3 notches cut into bottom edge of main body |
| Encoder centre-to-centre | 38.1 mm | 2× key spacing (2 × 19.05 mm) — symmetric about vertical centreline |
| Encoder knob edge-to-edge gap | 22.1 mm | 38.1 mm spacing − 16 mm knob diameter — comfortable clearance |
| PCB bounding box | 114.5 mm × 49 mm | Case interior minus 2.5 mm walls; custom non-rectangular silhouette |
| Key switch plate cut-outs | 14.5 × 14.5 mm | Standard 14.0 mm MX plate hole + 0.25 mm clearance each side |
| Encoder shaft holes | 7.0 mm ø | EC11 shaft 6.0 mm + 0.5 mm clearance |
| USB-C port cut-out | 10 × 4 mm | Centred on top edge of main body |
| Wall thickness — flat sections | 2.5 mm | FDM ASA structural minimum |
| Wall thickness — arm joints | 3.0 mm | Stress concentration point; increase to prevent layer-line cracking |

### Height  (Z-axis, desk to top)

| Layer | Thickness | Notes |
| --- | --- | --- |
| Bottom shell (FDM ASA) | 2.5 mm | Outer base wall |
| PCB standoff clearance | 3.0 mm | Room for solder joints and hotswap socket (1.8 mm) below PCB |
| PCB (FR4) | 1.6 mm | Standard PCB thickness |
| Top plate (FDM ASA) | 1.5 mm | Switches clip through plate cut-outs |
| — Case body subtotal — | 8.6 mm | Desk to plate surface — tray height |
| MX switch body above plate | 11.6 mm | Cherry / Gateron MX-compatible |
| OEM keycap (row profile) | 7.5 mm | Keycap sits on switch stem |
| — TOTAL product height — | 27.7 mm | Desk to top of keycap — comparable to Rama M6-B (26 mm) |
| Encoder knob above plate | 17.0 mm | 16 mm ø × 17 mm tall knurled aluminium knob |
| Encoder knob total height | 25.6 mm | Desk to top of knob — slightly below keycap top |

## Bill of Materials

| Component | Qty | Spec | Source |
| --- | --- | --- | --- |
| Case (top + bottom) | 1 set | FDM ASA, closest orange/red-orange filament to #DA7756 — confirm colour via test print first | JLCPCB 3D print |
| Custom PCB | 1 | Custom silhouette, 1.6 mm FR4, ENIG finish | JLCPCB PCB + PCBA |
| Pro Micro controller | 1 | ATmega32U4, USB-C preferred, drop-in | LCSC / AliExpress |
| Hotswap sockets | 5 | Kailh MX hotswap — PCB-mount | JLCPCB PCBA or pre-sourced |
| Diodes | 5 | 1N4148 SOD-123 SMD | JLCPCB PCBA |
| Rotary encoders | 2 | EC11 with detents, 20 mm shaft | LCSC |
| Encoder knobs | 2 | Black anodised aluminium, knurled, M6 | AliExpress / custom |
| Keycaps — 1U | 4 | Black PBT or ABS blank, MX stem | KBDFans / AliExpress |
| Keycap — 1.75U | 1 | Black PBT or ABS blank, MX stem (caps lock size — no stabiliser required) | KBDFans / AliExpress |
| MX switches | 5 | Buyer's choice — hotswap, any MX-compatible | Included or buyer-supplied |
| Braided USB-C cable | 1 | Black, ~1.5 m, USB-C to USB-A/C | Sourced separately |
| M2 screws | 4 | M2 × 6 mm, countersunk, black | Standard hardware |
| Brass heat-set inserts | 4 | M2 × 3.5 mm, for case assembly | Standard hardware |

## Production Path — JLCPCB

| PCB design | EasyEDA (native JLCPCB tool). Pro Micro footprint, 5 switch positions (4× 1U + 1× 1.75U), 2× EC11 encoder footprints, single-layer routing sufficient for this key count. Export Gerbers directly from EasyEDA to JLCPCB order. |
| --- | --- |
| PCBA assembly | Submit BOM + CPL files. Have JLCPCB populate hotswap sockets, diodes, EC11 footprints, and passive components. Pro Micro drops in after delivery — no SMD soldering required by hand. |
| Case 3D print | FDM ASA via JLCPCB 3D printing (same order as PCB). Print in closest available orange/red-orange ASA filament — confirm colour choice with a cheap flat test swatch before ordering the full case. Wall minimum 2.5 mm; 3 mm at arm joints. Specify 0.2 mm clearance on all mating faces. Top plate + base plate printed separately and assembled with M2 heat-set inserts. No post-processing required. |
| Firmware | QMK — well-supported for Pro Micro macropads with EC11 encoders. Flash via QMK Toolbox. Encoder map: left knob = volume/scroll, right knob = configurable. All keys user-remappable via VIA if VIA support is added. |

## Go-to-Market Strategy

1  Align with the original creator

DM them on X before building. Get explicit agreement — minimum: free unit + credit. Ideal: small per-unit revenue share so they have reason to actively promote, not just passively allow.

2  Build the prototype

Complete CAD, order colour test swatch + prototype PCB + case from JLCPCB. Confirm the ASA filament colour before the full case print. Goal: one physical unit to photograph — this is your entire marketing asset.

3  Coordinated reveal

Ship the prototype to the original creator. Agreed post: "I designed this — someone actually built it. Pre-orders open." Their existing audience becomes your launch audience.

4  Pre-order window

Take pre-orders via Shopify or equivalent. Set a firm MOQ and delivery timeline. Pre-order revenue fronts the manufacturing cost — no capital risk.

5  Production + fulfilment

Hit MOQ → full JLCPCB production order → assemble (drop in Pro Micro, flash firmware, fit switches + keycaps) → ship.

## Prioritised Next Steps

- DM the original X creator — align before committing time to CAD

- Finalise case CAD — robot silhouette, 3 mm walls, encoder + switch cut-outs, M2 insert bosses

- Design PCB in EasyEDA — Pro Micro footprint, 5× Kailh sockets, 2× EC11 footprints, USB-C passthrough

- Order FDM ASA colour test swatch — small flat print in orange and red-orange ASA, compare against #DA7756 in natural light to confirm filament choice

- Order JLCPCB prototype — PCB + PCBA + FDM ASA case in a single order once colour is confirmed

- Photograph finished prototype — this single asset drives all pre-order conversion

- Set up pre-order storefront — clear MOQ, timeline, and price listed upfront

Fan project — not affiliated with or endorsed by Anthropic. Claude and Claude Code are trademarks of Anthropic, PBC.